PC-Digital Storage Oscilloscope Adapter MARK-2

Version 3.3b

Copyright(C) 1991-2002 David L. Jones TRONNORT Technology

Notes on version 3.3b by Warren Stutt:

I have just realised that in the DSOA Mk2 hardware, that a possible internal

earth hum loop can be caused through the ground connections of J1 (Vertical

In) and J3 (External Trigger) if the sockets both have their grounds connected

to the metal front panel. This could be solved by replacing J3 with an

insulated socket.

The main reason I produced this version is so I could enhance the Waveform

(W) function to display XY traces (Lissajous patterns). However, I have also

made some other improvements.

David Jones kindly gave me a copy of the source code written in Microsoft

BASIC PDS 7.1 which I have ported to Borland Pascal 7. I have made some

optimistaions and fixed some minor bugs. Note that if you are compiling this

program, that I am using an updated CRT unit as decribed in the source.

The Waveform (W) function has been updated to select bewteen six waveform

modes, which are now:

1) Channel 1 active, Single trace as before

2) Channel 2 active, Single trace as before

3) Channel 1 active, Dual trace as before

4) Channel 2 active, Dual trace as before

5) Channel 1 active, XY trace (Lissajous patterns)

6) Channel 2 active, XY trace (Lissajous patterns)

For the XY trace, Channel 2 is the horizontal (X) coordinate and Channel 1 is

the vertical (Y) coordinate.

I have only tested the new version on DSOA Mk2 hardware with modifications as

described by David Jones in his readme.doc for the version 3.3a software

below, but it should hopefully be compatible with all the hardware that

version 3.3a is compatible with.

It is forwards and backwards compatible with version 3.3a TNNTDSOA.CFG, .DSC

and .WAV files although leading spaces on positive numbers have been removed.

This also applies to .ASC files.

The Files (F) function no longer displays the current directory or the free

disk space, however this allows more space for the files to be listed. Also,

if there are more files to list than what can fit on the screen, the first

files are listed instead of the last files as previously.

The font used is the same size, but different style.

I found the new version to have roughly comparable update speed to version

3.3a software when tested on a 450MHz Celeron PC and to be significantly

faster than version 3.3a when tested on a 25MHz 386SX PC. I believe I *could*

improve the speed further on both fast and slow machines if there is interest.

Found to have no screen flicker on either of the above machines.

It should now be more friendly to the printer when it is using the same

parallel port as the DSOA is configured for and when the plugs are swapped

between them.

I am happy for users to send bug reports to me at wstutt@xtra.co.nz

Regards,

Warren Stutt.

16th December 2002.

Following is the original contents for the readme.doc file included with the

version 3.3a software by David Jones.

==============================================================================

PC-Digital Storage Oscilloscope Adapter MARK-2

Version 3.3a

Copyright(C) 1991-1995 David L. Jones TRONNORT Technology

DISCLAIMER :

The  complete DSOA  package  (including  executable  programs,  and

documentation)  is provided AS-IS  without  any  warranty  expressed  or

implied.   The software is not guaranteed to work  with  every  possible

hardware/software combination.   The purchaser assumes full risk for the

use, correct or otherwise of this software package.

IMPORTANT INFORMATION

=======================

There  are several important modifications to be done  to  the  MK2

DSOA  unit in order to improve it's performance and  utilise  the  extra

features  of this version of the DSOA software.   Please  refer  to  the

"MODIFICATIONS TO THE MK2 UNIT" section further on in this document.

COMPATIBILITY

===============

Version  3.2 and above is now compatible with all  versions  of

the  PC-DSOA hardware, including :

-  The latest Mk2 project described in the May/June/July 94  issues

of Electronics Australia

-  The original DSOA described in the Feb 93 issue  of  Electronics

Australia.

- The Rev2.0 real-time hardware modification for the original DSOA.

Note  that some of the  new  features  in  the  software  are  only

operational with the new Mk2 design. This will be discussed later.

INTRODUCTION

==============

The DSOA project started with the first very popular design in  the

February  93 edition of EA.,  which was designed by TRONNORT Technology.

The  main feature of that design was the very low cost,  while providing

adequate performance.

Then came the Rev2.0 hardware modification and Ver2.0 software that

provided real time screen updating and automatic control.   Although the

software  provided a rudimentary calibrated graticule,  the  design  was

essentially  an uncalibrated one.   Triggering options  were  also  very

limited.

Many  constructors have added fully calibrated controls and  better

triggering  functions, similar to the new  DSOA  design.   However,  the

standard software never supported any of these add-on's, and many people

had to write their own software from scratch.

It  must be pointed out that the  DSOA  Mk2  project  was  designed

independently  by Jim Rowe at Electronics Australia,  and  not  TRONNORT

Technology,  although it is based on the original design.  It should not

be confused with the HSOA (High Speed Oscilloscope Adapter)  design that

was  mentioned in previous versions of this software,  as this design is

still under development at TRONNORT Technology.

Hereafter,  the word DSOA relates to  the  new  Mk2  design  unless

stated otherwise.

NEW FEATURES

==============

The latest DSOA design includes many of the most asked for features

that  were lacking in the original DSOA design.   The most notable being

calibrated  time/voltage controls and more  flexible  triggering  modes.

Maximum sampling speed has been increased to 1MHz,  and  the  input

amplifier bandwidth has been almost doubled.

These  new features meant that the software had to be  considerably

rewritten  to take full advantage of the new  features.   As  such,  the

program  interface  and  operation  is  quite  different  from  previous

versions.

The  following are a few major differences between  Version  2  and

Version  3 of the software.   Version 1 has very  little  similarity  to

either and will not be discussed :

1)  The most notable change would have to be the graticule. The new

graticule is a fixed 10x10 size, and provides dashed centre lines to aid

in  measurement.  The reason for the 10x10 size as opposed to  the  10x8

(HxV)  graticule on a normal oscilloscope,  is that  it  gives  a  round

number  for the Volts/Division setting.   The 10x10 size does  have  one

other  advantage in that the 10% & 90% reference lines are  not  needed.

This  makes for a clearer and less  confusing  (if  somewhat  different)

display.

The main graticule & waveform display area is considerably  smaller

than the previous version,  which was almost full screen.  This was done

to  allow the use of faster  display  algorithms.   It  also  makes  the

display look more like a regular oscilloscope.   Although it is smaller,

the  display actually provides a more accurate waveform,  as the display

does  not have to be scaled.   IE, the display now shows one sample  per

pixel in the vertical direction.

2)  A major difference for users of earlier model computers is that

the program will now only operate on an EGA or higher resolution screen.

The  program will always operate in EGA mode,  even on a VGA  or  higher

monitor.   EGA only operation was chosen  due  to  the  scaling  problem

described above, to allow page switching, and for a better font size.

3)  The calibrated time and voltage scales are  no  longer  entered

manually,  as the calibrated settings on the DSOA make  this  redundant.

Now  it is simply a  matter  of  selecting  the  required  timebase  and

attenuation settings,  which match the front panel settings on the DSOA.

Note,  that the DSOA front panel is calibrated in full scale voltage for

the vertical range,  and time per sample for the timebase. This may make

the  range selection a little confusing at first,  but the software also

displays  the same settings as the front panel and then  calculates  and

displays the corresponding division settings according to a normal CRO.

4)  The screen updating is now much  faster  when  displaying  many

samples.   For example,  updating the full 32K samples is  now  over  20

times  faster.  There is only a  small  improvement  when  updating  the

default  number of samples,  which has been changed from 640 to  500  in

this version.  The value of 500 was chosen to give a round value for the

Time/Division setting.

5) The display "window" now only zooms and expands in groups of 50.

There  is no longer any need to manually enter  the  number  of  display

samples  required.  Again,  the value of 50 calculates  nicely  for  the

Time/Division  setting.  Above 500 samples,  the minimum increment value

becomes 500.

6)  All of the current settings are displayed in three boxes on the

right  of the screen,  which makes it easier and quicker to identify the

current status.

7)  Due to popular demand, a print screen function is now included,

which  allows direct screen printout to Epson,  IBM, DeskJet &  LaserJet

compatible printers.  These four modes should cover almost every printer

available, as any printer worth its salt will emulate one of these.

8)  Screen flicker in UPDATE mode  has  now  been  almost  entirely

eliminated,  with no loss of update speed.   A fast 486 machine  with  a

fast  local bus video card can update the screen at over  25  times  per

second!.   On some very fast machines a small flicker of  the  graticule

may  be visible,  this is due to the update rate of the video card.   It

will differ with a different number of displayed samples.   There is not

much that can be done about this, as it is machine dependant.

Installation & Hardware Requirements

=====================================

Hardware requirements for the DSOA are:

1) An IBM compatible computer with at least a 256KB EGA graphic card

2) 256KB  of free memory.

3) A standard PC compatible parallel port at base address 378h,

278h, or 3BCh.

4) An optional Epson, IBM, DeskJet, or LaserJet compatible printer.

The program will operate entirely from a floppy drive. However, the

disk must NOT be Write-Protected.

The executable program DSOAxx.EXE (where xx is the version  number)

is the only file required for operation.   However, the program needs to

load  and save the configuration  file  "TNNTDSOA.CFG"  in  the  current

directory.   If this config file is not available, then the program will

use default settings on boot-up.

For users with monochrome monitors,  start the program with the 'M'

switch. Eg, "DSOAxx M". Alternatively, run the DSOAM.BAT batch file.

Start the program by executing the "DSOA.BAT" file.

The  installation batch file "INSTALL.BAT" will copy all the  files

on the disk into a directory called DSOA3x on the specified drive.  The

installation  batch file does not do anything special,  so the files can

be copied manually for those that prefer to do it that way.

If  the program is to be operated from a floppy drive,  then make a

backup  of the original DSOA diskette  and  use  that.   NEVER  use  the

original disk!.

USING Version 3 SOFTWARE WITH THE ORIGINAL DSOA

=================================================

This  version 3 software is compatible with the original DSOA,  but

many of the new features will not work due to the original hardware  not

having  support for them.   The features that NOT  compatible  with  the

original DSOA include:

-  Update  &  One  shot  modes  only  work  with  the  Rev2.0   hardware

modification.

- Data logging and Clock rate selection.

- Stop  mode. Software  must always be set to Start  mode.

-  The calibrated range settings are not applicable unless the  controls

on the DSOA are set with a calibrated signal.

The  'Calibrate' function and scalable grid in Version 2.0 are  not

directly included in the new Version 3.xx software.  This is because the

scalable grid caused to many confusions,  and the Calibrate function was

rarely  used (and provided too many significant figures).   However, the

new  range settings and fixed graticule provide similar features if  you

wish  to manually calibrate the  original  DSOA,  by  sampling  a  known

reference signal.

The interpolation function is no longer selectable, as it does not

provide any real benefit, and also slows display updating.

PROGRAM OPERATION

===================

When the DSOA program is started,  it will display the main screen.

This screen,  which includes the waveform display area and menu options,

contains all the information and options that are needed to operate  the

program.   There are no extra  hidden  menus  or  screens,  which  makes

program operation quite straight forward.

The  screen is roughly divided into three parts:-

1)  The top left of the screen is the waveform display area,  which

takes up about two-thirds of the screen.   It is initially overlaid with

a 10x10 graticule (which can be turned off).

Directly  below the waveform area is a thin  rectangular  box  that

runs  the length of the waveform area,  called the window position  box.

The length of the box represents the currently set RAM Buffer size,  and

the  highlighted portion  within  the  box  represents  the  amount  and

position of the currently displayed waveform with respect to the Buffer.

2)  The top right of the screen contains  four  boxes,  the  bottom

three of which display the status information and settings.

The CH1 and CH2 boxes display the current settings for Channels 1&2

respectively.  The word "ACTIVE" is displayed beside the current channel

that  will be used for sampling and file operations.   In SINGLE channel

mode  the ACTIVE setting also decides which channel will  be  displayed.

However, in DUAL channel mode, the ACTIVE setting does not influence the

waveform display.

The settings displayed in these two boxes are as follows :

FS  Vt :- The total display voltage that matches  the  front  panel

VERT.RANGE setting on the DSOA.

T/Smp :- The length of time between each sample.   Should be set to

the same value as the TIME/SAMPLE setting on the DSOA.

V/Div :- The number of volts per vertical graticule division,  just

like on a standard CRO.

T/Div  :- The length of time  per  horizontal  graticule  division.

Again, identical to a standard CRO.

The  lower and largest of the four boxes contain  general  settings

that are common to both channels. The various settings are as follows :

Start  :- The starting position of the current window with  respect

to  the buffer size,  as displayed on the window position box.   IE, the

time between the highlighted bar and the start of the buffer.

Trace :- Toggles between "Single" and "Dual" trace mode.

#Samples :- Displays the number of samples that are currently being

displayed  in the window.   This will also be the number of samples that

are updated on the screen in Update (or real-time) mode.

Buff.Size :- The currently selected RAM buffer size.

Trig Mode :- Toggles between START and STOP trigger mode.

Trig From :- Selectes between triggering from the DSOA or the HOST

Port Addr :- The active DSOA port address.

Host Clock :- HOST clock sample rate in seconds.

3)  The bottom third of the screen contains the  available  program

options.   They are organised into four vertical rows.   The highlighted

first  letter of each command is the key that must be pressed  to  start

that particular command.

Following is a detailed description of each command :

Update Mode (U) :

=================

Enters real-time screen updating mode.   In this mode the DSOA acts

like  a normal oscilloscope and updates the  screen  repetitively  until

told to stop.

While  within update mode,  it is not possible to select any  other

commands. The only thing that is allowed is to press ESC to exit.

There  are three things that happen in update mode.   First, if the

TRIGGER  option is set to HOST then the software will trigger  the  DSOA

into  sampling, otherwise it does nothing.   Next it waits for the  DSOA

unit  to finish sampling,  and thirdly retrieves the number  of  samples

that are currently displayed.   The screen is then updated with this new

data  and the entire process will repeat itself.   The result being that

the repeatedly updated screen will emulate an analogue oscilloscope.

The  screen update rate in  this  mode  depends  on  the  following

factors :

1) Number of samples displayed. (Less samples = Fatser speed)

2) Being in START or STOP mode. (STOP mode is MUCH slower)

3) The sample rate of the DSOA. (Faster the better)

4) The hardware buffer size of the DSOA. (4K is fastest)

5) General computer and video speed. (PCI BUS Pentium = fastest!)

For  START mode,  if the current start position of the  display  is

greater than zero, then it is reseted to zero before Update mode begins.

For STOP mode,  the window will be moved to the end of the buffer.  This

is done so that the data just before the trigger point is displayed.  As

this is the whole idea of STOP mode.   STOP mode slows down the updating

significatly because all of the buffer data must be read in.

Updating in STOP mode will not work if the buffer size is less than

32KB. This is a DSOA hardware limitation. The software will display an

error message if the buffer size is not set to 32K.

A  slash character will be displayed in the bottom left  corner  of

the  screen and will toggle between "/"  and  "\"  when  the  screen  is

updating.   This will let you know if the triggering is  working,  which

allows screen updating to occur.

Pressing  the ESC key will tell the software to finish the  current

cycle,  and then retrieve all the data from the DSOA.  The data can then

be examined from the main screen.

One Shot Mode (O) :

===================

Exactly  the same as Update mode except that  the  screen  is  only

updated  once.  If the TRIGGER mode is set to DSOA,  then  the  software

waits  for the DSOA unit to trigger.   In HOST TRIGGER mode the software

generates a trigger pulse the moment the One Shot option is selected.

All of the data is retrieved after the DSOA has finished  sampling.

But only the number of selected samples are updated on the screen.

Data Logging (D) :

==================

Data  Logging mode is very  similar  to  One-Shot  mode  with  HOST

(asynchronous) triggering. Except that the PC (HOST) controls the sample

clock.   The period of which must be first specified with option 'C' (in

seconds). Synchronous triggering is not available in this mode.

When  this mode is selected,  the currently active data buffer will

be cleared.   The screen will only be updated once the number of samples

taken  has reached the current Buffer Size,  or the ESC key is  pressed.

Pressing  any other key while sampling is taking place  will  cause  the

software to imeadiately take the current sample reagaurdless of the time

period set.

Ensure that the sample rate switch on the DSOA is set to HOST  when

using the data logging feature.

Load File (L) :

===============

Retrieves the user specified ".WAV" file from the current directory

on  the current drive,  and stores it into the ACTIVE waveform.   Do not

enter  the ".WAV" extension,  only the 8 character filename.   Once  the

file has been loaded, the description of the waveform is displayed.

Only ".WAV" files can be loaded.   The ".ASC" ASCII files cannot be

loaded back into the program.

Save File (S) :

===============

Writes the currently ACTIVE waveform to a specified ".WAV" file  on

disk.   Any previous file of the same name will overwritten. You will be

asked  for a description  of  the  waveform,  which  can  be  up  to  60

characters in length.

Configuration  and calibration information about  the  waveform  is

saved into a file with the extension ".DSC".

Files (F) :

===========

Displays all of the ".WAV" files in the current directory.

Quit (Q) :

==========

Quits  the  program  without  prompting  the  user.   The   current

configuration  and   calibration   information   is   saved   into   the

"TNNTDSOA.CFG" file.  These settings will be automatically reloaded when

the program is next executed.

Retrieve Data (R) :

===================

Reads  in all of the data from the DSOA  buffer  upto  the  address

specified  by the Buffer Size setting.   The active channel is  the  one

used  to store the data.   Once the data has been uploaded, the waveform

will be redrawn with the new data.

In general use this command should not have to be used, as the data

is automatically uploaded after Update or One-Shot mode is used.

The data retrieved in STOP mode may not be valid with this command.

This  is because the software has no way of knowing if the  RAM  address

counter on the DSOA is still pointed to the start of the data.  Although

it  would work some of the time,  the validity of  the  data  cannot  be

guaranteed.

Buffer Size (B) :

=================

Specifies the size of the RAM in the DSOA.   Select between 4K, 8K,

16K, and 32K. This setting must be set to 32KB for STOP mode to work.

For those who have added the optional RAM size switch to the  DSOA,

this setting must be changed to reflect the current switch setting.

Address Select (S) :

====================

Sets  the parallel port address used  for  communicating  with  the

DSOA. The three options are :

LPT1: Base address 378hex

LPT2: Base address 278hex

MDPA: Base address 3BChex

Waveform (W) :

==============

Selects bewteen the four waveform modes available, which are :

1) Channel 1 active, Single trace

2) Channel 2 active, Single trace

3) Channel 1 active, Dual trace

4) Channel 2 active, Dual trace

The  active channel is the one which is used for storing data  from

the DSOA, and reading and writing to the disk.

Although  the DSOA  does  not  have  dual  channel  faculties,  the

software  allows two waveforms to be stored  in  seperate  channels  and

displayed  on the  screen  at  once  (Dual  trace  mode).   This  allows

rudimentry waveform comparisons.

The  waveforms are displayed in different colors  which  match  the

corresponding color of the information box.

Trigger HOST/DSOA (T) :

=======================

If  set to DSOA,  then the software will NOT trigger the DSOA  into

sampling  while in Update or One-Shot modes.   In this case the  trigger

signal  must come from the DSOA.   The trigger can come  from  Internal,

External,  or the Manual Trig button.   In Update or One-Shot modes, the

software  will just wait for the End Of Buffer  signal  from  the  DSOA.

Therefore  in this case,  no trigger = no EOB signal. So the screen will

not be updated.

Providing  a stable repeatable trigger  point  can  be  found,  the

screen  will be updated just like a  normal  CRO.   With  the  displayed

waveform  steadily displayed on the screen.   "DSOA" trigger mode is the

one  to use if you are measuring basic repeatable waveforms,  such as  a

sine  wave.  Although the DSOA  hardware  does  not  have  the  advanced

triggering  features off normal  CRO's.   Therefore  complex  waveshapes

(although repeatable) may not be displayed steadily.

In  HOST trigger mode,  the software will  trigger  the  DSOA  into

sampling the moment it finishes retrieving the current data.  This means

that the triggering signal will be totally asynchronous to the  measured

signal.   This will not  provide  a  stable  display,  but  instead  the

waveform  will "shift" across the screen.   But unlike a normal  CRO  in

asynchronous  mode, you will still be able to breifly see each  waveform

sweep on the display.

HOST  Trigger mode should only be used  if  the  screen  cannot  be

updated  in DSOA trigger mode.   It will also allow you to  display  ANY

type of signal, repetitive or not.

Graticule ON/OFF (G) :

======================

Toggles  the waveform graticule (grid)  on and  off.   Turning  the

graticule  off will slightly improve the update speed  while  in  Update

mode.

Mode Start/Stop (M) :

=====================

This  setting must be set to reflect the  Trig.Action  (Start/Stop)

switch on the DSOA.

Start  position is what is called "Post Triggering" mode.   Eg, all

the  data you see on the screen was sampled  AFTER  (POST)  the  trigger

point. IE, sampling STARTs when the trigger point occurs.

Stop  position is also called "Pre Trigger" mode.   Eg, all of data

was  sampled BEFORE (PRE)  the trigger point.   In the case of the DSOA,

only 100% pre-trigger is available.  IE, 100% of the data seen is before

the trigger point.

Stop mode is quite slow in Update mode,  due to the software having

to  retrieve all of the data in the buffer reguardless of the number  of

samples displaed.

NOTE  : In Ver 3.3,  STOP mode has a 1  second  delay  added  after  the

software  triggers the unit into sampling.   This is to ensure that  the

entire buffer fills with continuous pre-trigger waveform data.   If this

is  not done then the DSOA could be triggered and hence  stops  sampling

just  after STOP mode is started.   The result is that only a portion of

the buffer contains correct data from this capture, the rest of the data

is left over from the any previous capture(s).  Thus this feture is only

effective at high sample rates.

Clock (host) rate (C) :

=======================

Sets  the sample clock rate (in seconds)  for  Data  Logging  mode.

This option must be set before entering Data Logging mode.

Print Display (P) :

===================

Prints  the screen to a compatible printer.   If you only have  one

parallel  port and have to use the same port for the printer  and  DSOA,

then it is best to quit and reload  the  DSOA  program  after printing.

You  will  first  be  asked  to  select   between   Epson/IBM   and

DeskJet/LaserJet  modes. If DeskJet/LaserJet mode is selected  then  you

will  also be asked for the required resolution in Dots  Per  Inch,  and

wether  or not compressed mode is used.   Compressed mode  generates  an

almost square printout, instead of the usual rectangular screen shot.

The bottom menu options are not included on the printout,  as these

are  not needed.   The current DOS date and time are also included as  a

reference.

Printing can be canceled by pressing the ESC key at any time.

Export to ASCII (E) :

=====================

Works exactly like the Save command,  except that the data is saved

in  ASCII format which can be imported into other packages for  anaysis.

The description file is NOT saved in this mode.

Scroll window ([]) :

====================

Moves  the highlighted 'window' portion of the  waveform  left  and

right in increments of the current windows size.

ENTER must be pressed to redraw the window.

Fine window scroll (<>) :

=========================

Moves  the highlighted 'window' portion of the  waveform  left  and

right  in increments of 50 samples.   In the default 500 sample  window,

the waveform will scroll across one division at a time.

ENTER must be pressed to redraw the window.

Expand/Zoom window (+-) :

=========================

Expand  and Zooms the size of the current 'window'.   Or  in  other

words, the number of samples that are displayed in the window.

For  500  samples  and  below,  the  number  of  samples  jumps  in

increments of 50. Above 500 samples, the increment value is 500.

ENTER must be pressed to redraw the window.

Window Limits (Home/End) :

==========================

Moves the highlighted window to either end of the buffer.

ENTER must be pressed to redraw the window.

Vert.Range (Up/Down arrows) :

=============================

Selects the Verticle range of the DSOA.   This setting must reflect

the  current setting on the DSOA,  in order for the  calibration  to  be

accurate. Values range from 1V to 50V.

Time/Sample (Left/Right arrows) :

=================================

Selects the sample rate of the DSOA.  This setting must reflect the

current  setting on the  DSOA,  in  order  for  the  calibration  to  be

accurate. Values range from 1us to 1ms.

Redraw Window (ENTER) :

=======================

Redraws  the display window using the current settings.   This must

be  done after most operations involving the display  window.   When  in

doubt about the validity of the display, press ENTER !.

========================================================================

========================================================================

Version 2.0 Hardware Modification  (For original DSOA only)

=============================================================

The  following information  should  be  sufficient  to  enable  the

original  DSOA to be modified  to  version  2.0  status.   This  is  not

applicable  to the Mk2 design which already has these  capabilities  and

more.  Without this modification, the version 3.2 software will not work

in  real time update mode,  or One shot mode.   Please excuse the  crude

ASCII  diagrams, extended ASCII doesn't exactly have  a  diode  symbol!.

The diagrams show exactly what is explained in the text.

All  diodes mentioned in the following  text  are  general  purpose

1N914/1N4148  silicon switching diodes.   Any equivalent type should  be

suitable.  Wiring can be done with suitable lengths of light duty hookup

wire. It does not have to be sheilded.

STEP 1: Cut one leg of R4 (10k) from the PCB. Or remove it all together.

STEP 2: Cut one leg of R5 (10k) from the PCB. Or remove it all together.

STEP 3:

Connect the  anode end of  a diode  to  pin 8 of  the DB25 parallel

connector. Connect the  cathode end to the signal  side of Reset switch

S3 (not the  Vcc side, but the one  that goes to pin 4  of IC8a). It is

best to mount the diode on the switch.

Vcc    S3 Reset      D1

���     �����       1N914

������-�   ����������<���������� Pin 8 DB25

�   k   a

IC8a

STEP 4:

Connect the  anode end of  a diode  to  pin 7 of  the DB25 parallel

connector. Connect the  cathode end to the signal  side of Start switch

S4 (not the  Vcc side, but the one  that goes to pin 6  of IC8a). It is

best to mount the diode on the switch.

Vcc    S4 Start      D2

���     �����       1N914

������-�   ����������<���������� Pin 7 DB25

�   k   a

IC8a

STEP 5:

Disconnect all wires from S1,  the Store/Read switch. This needs to

be completely rewired. On both poles of the switch, connect the cathode

ends of  TWO diodes to  the 'throw' terminal  (the outer contact)  that

equates  to the  'Store' switch  position. Use  two separate diodes for

each  pole (4  diodes total).  The other  anode end  of one  of the two

diodes from each switch pole is connected to pin 9 of the D25.

There should now be one diode left on each switch pole. Connect the

anode  end of one of these diodes (either one),  to the 'EOB' wire (from

pin  8, IC2)  that was originally connected to this switch.   Also, take

this 'EOB' wire to pin 15 of the D25.  The common terminal of the switch

pole that was connected the 'EOB' wire,  should be connected to the 'CS'

wire (from pin 13, IC5) that was originally on the switch.

The last anode end of the remaining diode is connected to the 'INT'

wire (from pin 9, IC5) that was originally on this switch. The remaining

common terminal of this switch pole is connected to the 'R/W' wire (from

pin 1, IC9a), that was originally from this switch.

The two remaining 'throw' terminals of the switch poles that equate

to the 'Read' switch position,  should be tied together and then tied to

Vcc via a 10K resistor.

D3

1N914

��������<��������� Pin 9,DB25

�

Store       �    1N914         EOB

CS      S1    O�����������������<��������� Pin 8,IC2

Pin 13,IC5 �����O�����         �      D4   �

O         � RM2       �

Vcc                  Read �         � 2K2�      ������ Pin 15,DB25

���                       �         �

�                        �        ���

� RM1                    �        Gnd

� 10K�                   �

�                        �                D5

��������������������������              1N914

�                                  ��������<����� Pin 9,DB25

�                                  �

�                      Store       �    1N914     INT

�         W/R      S1    O�����������������<����� Pin 9,IC5

�    Pin 1,IC9 �����O�����         �      D6

�                        O         � RM3

�                   Read �         � 2K2�

��������������������������         �

���

Gnd

How the Modification Works

============================

The following section is for those people who are curious about how

the modification works.

The modification is quite simple and merely involves the PC  taking

over control of the Store/Read, Reset, and Start switches on the DSOA.

Diodes  D1 & D2 emulate the Stop  &  Start  switches  respectively.

When  the software sets pins 8 &  7  on  the  parallel  port  high,  the

corresponding  diode  is  forward  biased  across  the   switch.    This

effectively closes the switch.  When pins 8 & 7 are low (or the cable is

disconnected),  the diodes are reversed biased and the switches  operate

normally.

The  two resistors on the PCB (R4 & R5)  must be removed, otherwise

they  will form a voltage divider with the new resistors RM2 & RM3  when

S1 is in the 'Store' position.   This is undesirable.  When S1 is in the

'Read' position,  the 'CS' & 'R/W' lines will be tied high via RM1. This

disables the ADC and puts the SRAM chip into read mode.

For  normal operation,  S1 must be in the 'Store' position. In this

case,  the two diodes and  pull  down  resistor  for  each  switch  pole

(D3,D4,RM2 & D5,D6,RM3) act as an OR gate. If the software sets pin 9 of

the  parallel port LOW,  then this effectively sets one of the  OR  gate

inputs LOW.   The 'EOB' & 'INT' lines are feed straight though to 'CS' &

'R/W'  respectively.  This is the same operation as the  original  DSOA.

Now  if pin 9 is set HIGH by the software,  this pulls both 'CS' & 'R/W'

HIGH.   This is the same as switching S1 to 'Read' mode.  Therefore, the

software can effectively toggle S1 by toggling pin 9!.

As  can be seen,  if the software sets all of the new pins  (7,8,9)

LOW (or if the cable is disconnected),  then the DSOA operates as if the

modification wasn't there.  As it so happens, the old DSOA software (Ver

1.2)  sets these pins low.  Therefore, the old software will still work.

For the new software, just keep S1 set to the 'Store' position.

The  'EOB' (End Of Buffer)  signal  is  also  feed  back  into  the

parallel  port via pin 15.   This allows the software to detect when the

DSOA  has finished sampling,  in which case it can start reading in  the

data.

MODIFICATIONS FOR THE MK2 UNIT

================================

The  following modifications to the MK2  unit  are  not  necessary,

however  the MK2's performance will be greatly enhanced with the use  of

these  modifications and the  Ver  3.3  software.   If  you  are  having

problems with the MK2 unit, these mods may provide a solution :

NOTES & ERRATA :

================

The  following note appeared in the Notes & Errata  column  in  the

September  1994 edition of EA.   It may not have been included with  the

kit :

The  link on the bottom of the PCB that is shown as  connecting  to

pin 9 of U9b (Q-bar) should actually be connected to pin 8 of U9b (Q).

Note that an enhanced version of this mod is described below.

FURTHER PROBLEMS ENCOUNTED :

============================

1) PROBLEM : Triggering is unstable in START mode.

SOLUTION: Bypass R28 with a 100pF-220pF cap.

2) PROBLEM :  General unstable triggering

SOLUTION:  This is a very important modification,  as  the  DSOA  MK2

design  has  always  had  a  very  small  trigger  "window".   This  was

a compromise in the hardware to allow use of START and STOP modes. To

overcome this trigger limitation, carry out the following mod :

- Replace the START/STOP switch (S6)  with a DPDT type.

- Wire one pole of S6 (now S6a) as per the original schematic.

- Disconnect the mod link on the bottom of the PCB.

- Connect pin 15 of J2 (one end of the old wire mod) to the common

terminal of S6b.

- Connect pin 8 of U9b to the STOP side of S6b.

- Connect the EOB line (pin 1, U10a) to the START side of S6b.

The  purpose of this modification is to allow  the  END  OF  SAMPLE

signal  to come from the EOB line in START mode,  or the Q-bar output of

U9b  in STOP mode (the same as the old mod link).   This makes sure that

the  software correctly detects the end of sampling when in START  mode.

Before,  in DSOA TRIGGER MODE if the trigger  did  not  occur  within  a

certain "window" in the software,  then the software will think the DSOA

has  finished sampling and will read  in  the  waveform.   However  this

waveform  will not have been updated,  and thus  the  software  is  just

re-reading  the previous waveform.   Before, you had to be lucky to have

the trigger point occur within the trigger window,  but now the software

will always wait for the DSOA to finish sampling.

This  modification will only be partly sucessful with the old  Ver

3.2 software, however it is still a very worthwhile mod if your staying

with Ver 3.2.

3)  PROBLEM : Vertical inaccuracy approaching full scale.

SOLUTION:  Replace R35 with a link.  This "stopper" resistor forms a

voltage divider with the feedback resistors in the circuit of U8b  which

degrades  the vertical accuracy of the input amp.   U8a is working as  a

stable  rail source,  and thus  R35  effectively  increases  the  source

resistance of this rail to 1K�.  Replacing it with a link eliminates the

problem.

4)  PROBLEM :  Erratic address incrementing (particulary evident in STOP

mode). Software not reading exactly 32K samples.

SOLUTION:  Multiple clocking is occuring in the HOST  MEM  INC  line.

Bypass pin 13 of U10d to ground with a 47pF cap.

NEW FEATURES IN VERSION 3.3a

==============================

- Improved support for operation under Windows.

- Improved support for high speed machines.

- Correction of possible data sample corruption.

- Correction of possible problem in STOP mode.

- Problem with Data Logging triggering has been fixed.

- Correctly restores original port values after use.

- Improved waveform toggling command.

- Support for new triggering modification allows far better triggering.

========================================================================

========================================================================

Advertisement!

EA Eprom Programmer

=====================

If  you have built,  or  are  thinking  about  building  the  Eprom

Programmer project published in EA September/October 93, then read on!.

I  designed a similar but more powerful version of this  project  a

few  years ago,  but never got around to getting it published.  Hence, I

know  a fair bit about such things.   The project hardware described  in

the article does work reasonably well,  but the hardware is only as good

as the software.   The GW-BASIC software in the article isn't even worth

typing  in, and the new 'c' software available from the  author  doesn't

provide  many  of  the  features  that  are  expected  from   commercial

programmers.

The programmer software called PCEB (I love four letter  acronyms!)

is  just as good as any commercial programmer software,  and taking into

account  the hardware I had to work with it has most of their  features,

some of which are :

Complete  HEX/ASCII editor,  Save,  Load,  Auto  Program,  Compare,

Verify  and Blank Check.   A customizable  device  type  library  allows

editing  of  device  type's  that  aren't  supported  by  the   supplied

protocols.   Support for 0.1ms pulses,  and compatibility  for  machines

from XT's to Pentiums!.  A colourful menu driven interface operationally

compatible  with many commercial programmer interfaces,  and support for

devices  up to  16Mbit  in  size.   CRC  checksum  calculation  is  also

included.

Complete  documentation and the entire  commented  source  code  in

Pascal 7 are included on the disk.

The  programmer software should be available from Altronics by  the

time you read this.   For DSOA owners, it can also be purchased directly

from  TRONNORT Technology for $13 (cheque or money order).   Be sure  to

specify  the disk size required.   Send all orders and enquiries to  the

address below.

Final Word :

============

If  you have any problems with  the  software,  or  ideas  for  any

improvements  then write to me and let me know.   To receive the  latest

information on new products and upgrades, be sure to fill out and return

the registration form, which should have been included with the package.

David L. Jones

TRONNORT Technology

12 Copeland Road

Lethbridge Park NSW 2770.

April 2nd, 1995.