Analog Interface Unit (AIU) Rev 1.0

=====================================

Designed by, David L. Jones

TRONNORT Technology

Documentation Rev 1.1 : 7/9/97

Copyright & Disclaimer :

========================

The AIU hardware design and all associated software is Copyright (C) 1995

by David L. Jones of TRONNORT Technology.

Although  great care has been taken to ensure compatibility  and  correct

operation,  all  software  including  driver   routines,   examples   and

documentation  are supplied AS-IS.   They are not guaranteed to work with

all  possible hardware/software combinations,  nor are they guaranteed to

be suitable for any particular application.

The  AIU software driver routines & AIU hardware shall not be  copied  or

distributed in any whole or modified form to any third party.

Features :

==========

The AIU has a range of features which include :

- Powered entirely from the PC's parallel port

- 'Dongle' size unit that can be connected directly to a parallel port

- Eight bit resolution ADC and DAC.

- 1M� ADC input impedance allows use of standard x10 adapters.

- Stable onboard voltage reference.

- True bipolar input and output ranges. Onboard negative rail generator.

- Selectable ranges for input and output : +/- 2.5v and +/-1.25v

- Lever terminal block for input/output connections.

- Complete easy to use driver routines in BASIC, PASCAL, and C.

- Low cost.

Overview:

=========

The AIU was designed to fulfill a need for a general purpose analog  port

that  would allow a standard PC to interface to an analog world.   It  is

eminately  suitable for applications such as datalogging (short and  long

term),  data acquisition,  and connection to sensors and transducers etc.

It  is suitable for  almost  any  application  that  requires  an  easily

customizable 8 bit ADC and/or DAC.

Considering the specifications of the hardware, and the fact that the AIU

relies  on the PC to do real time  processing,  it  was  decided  not  to

include  application  software  such  as  an  oscilloscope  and  spectrum

analyser.   Not only do these applications take a great deal of  time  to

write,  but they would be little more than toys with  a  sample  rate  of

<50KHz.   There are  other  commercial  units  available  with  wonderful

general  purpose graphical application software,  but what good is a 5KHz

oscilloscope ?.

The  AIU  has  been  designed   specifically   for   custom   development

applications,  and complete driver routines are provided in BASIC, Pascal

&  C.  All routines are DOS based.   These routines  allow  the  user  to

easily  create  customized  applications  for  the  AIU  under   multiple

programming languages.

CONSTRUCTION :

==============

For those who have purchased the AIU as a kit, the following section will

aid  in the assembly and testing of the AIU.   If the AIU  was  purchased

already assembled, then this section can be skipped.

Construction  is relatively straight forward,  and the supplied component

overlay  diagram along with the schematic should be sufficient  to  allow

correct assembly.  However, the following notes may help :

Start  by installing all of the resistors.   All but  two  of  these  are

mounted  vertically. Next install the four wire links which can  be  made

from the pins cut from the resistors.   The two diodes and six capacitors

can  now be installed,  but watch the polarity of the  diodes  and  three

tantalum  caps.  The five IC's can be now installed.   Be sure to observe

proper antistatic handling precautions with the IC's,  and make sure that

the IC's are inserted in the correct positions and orientation as per the

component overlay.  No sockets are provided with the IC's as it makes the

unit  look ungainly and provides less  reliability.   The  D25  connector

should now be installed.   The hex nuts from the D25 have been removed to

allow  the AIU to be connected directly to the parallel  port  connector.

The  final  component  is  the  four  way  terminal  block  for  the  I/O

connections.   This terminal block should be glued to the PCB to  provide

adequate strength when operating the levers.   Super glue can be used for

this  purpose, but allow it to dry  before  soldering  the  pins  of  the

connector.

After  assembly the AIU will be  already  calibrated  and  ready  to  go.

Connect  it to the PC's parallel port and  run  the  AIUTEST.EXE  program

supplied on the disk.  This program will first detect the AIU and display

what port it is connected to.  It will then proceed to test the data port

for correct operation.  The next test the program performs will require a

loopback connector on the input and output of the AIU. Each range is then

tested for all possible voltage values.

If the AIU is not detected then the most likely problem will be with  the

unit's construction,  however, the AIU should also be tried on another PC

or  parallel port if at all possible to eliminate the possibility  of  an

incompatible parallel port.

HARDWARE PROBLEMS :

===================

Due to the non-standedness of the PC's parallel port,  some ports may not

be able to supply the current and/or voltage requirements for the AIU. If

you are experiencing problems,  measure the +5v and -5v rails on the AIU.

They should be within +/-0.25v. If this is not the case then the parallel

port  being used could have bipolar outputs which  may  not  be  able  to

supply  the voltage requirements for the AIU.   If this is the case, then

the only thing that can be done is to provide an external +5v supply  for

the  AIU, which can be connected directly to  the  +5v  rail.   The  link

connected to R1-R5 must be removed if using an external supply.

SOFTWARE DRIVER ROUTINES :

==========================

AIU  driver routines are provided for DOS in BASIC,  Pascal and C format,

which  will enable anyone to easily create customized applications  under

the  DOS environment.   The BASIC  routines  are  fully  compatible  with

Microsoft  QBASIC which is supplied with DOS versions 5 and above.   This

will allow the creation of customized applications without the need for a

compiler.

The driver routines have been tested with the following languages :

BASIC : Microsoft QBASIC

BASIC PDS 7.1

Visual BASIC 1.0 for DOS

QuickBASIC 4.5

PASCAL : Borland Turbo Pascal 7.0

C : Borland C++ Version 3

Other  versions of the above products may  require  modification  to  the

driver  routines to enable correct operation,  however, these changes are

likely to be minor.

The  driver routines for  all  three  languages  contain  functions  with

identical  names and parameters (were possible by each  language),  which

allows  pseudo code and algorithms to be easily ported from one  language

to another.

The BASIC routines are provided in a separate file called AIU10.BAS which

can  be loaded as a secondary file within QuickBASIC,  BASIC PDS 7.1, and

VB  for DOS.   For QBASIC, this file will have to be merged with your own

program into one file.

The  Pascal routines are supplied in a unit file which allows  it  to  be

easily incorporated into any program by adding "AIU10" to the USES clause

at the beginning of your program.   The Example programs show how this is

done.

The  C routines are supplied in a header file called AIU10.H.   This file

can be included at the beginning of your program by adding

#include<aiu10.h>  to the start of your program.   An example program  on

disk shows how this is done.

Here is a description of each routine and their parameters :

ReadByte()

==========

DESCRIPTION :

=============

This routines returns the current byte on the data bus of the AIU. It can

be used to read back and verify a byte that has just been sent to the AIU

prior  to outputting it to the DAC.   It is  also  used  by  the  ReadABS

routine to read back a byte from the ADC after conversion.

PARAMETERS :

============

None.

RETURNS :

=========

Returns an integer value which is the same as that read from the data bus

on the AIU

ClockData(Bit)

==============

DESCRIPTION :

=============

Shifts  a single bit into the AIU's data register.   It is  used  by  the

SetByte routine to output a whole byte onto the AIU data bus.

PARAMETERS :

============

Bit ::  An integer value that acts as a boolean value. If Bit is equal to

zero then the output bit will be zero, else the output bit is one.

RETURNS :

=========

Nothing

SetByte(Value)

==============

DESCRIPTION :

=============

Sends  a byte to the DAC on the AIU,  which outputs the voltage specified

by that byte.  This routine uses the ClockData routine to output each bit

to the AIU's data register.

PARAMETERS :

============

Value :: The byte that is to sent to the DAC

RETURNS :

=========

Nothing

ReadABS()

=========

DESCRIPTION :

=============

This routine triggers the ADC on the AIU into sampling & then returns the

byte from the ADC. This routine uses the ReadByte routine to retrieve the

ADC data from the AIU.

PARAMETERS :

============

None.

RETURNS :

=========

Returns an integer value that is the value sampled from the ADC.

ReadVoltage(Range)

==================

DESCRIPTION :

=============

Triggers  the ADC into sampling and returns a floating point  value  that

represents  the exact voltage measured by the ADC.  This routine uses the

ReadABS routine to sample and return the byte from the ADC. This value is

then converted into a voltage reading depending on the Range parameter.

PARAMETERS :

============

Range ::  A boolean value that determines which range the routine uses to

calculate  the voltage.   0=+/-1.25v, and <>0=+/-2.5v.   Note, this range

parameter  does NOT change the actual range on  the  AIU.   It  has  been

assumed that this range has already been set using the SetRange routine.

RETURNS :

=========

The routine returns a floating point value representing the ADC voltage.

Power(OnOff)

============

DESCRIPTION :

=============

Turns power to the AIU on and off.

PARAMETERS :

============

OnOff  :: A boolean value.   If OnOff=0 then power is  turned  off.   Any

other value will turn power on.

RETURNS :

=========

Nothing.

SetRange(Range)

===============

DESCRIPTION :

=============

Sets  the DAC and ADC voltage range on the  AIU  to  either  +/-1.25v  or

+/-2.5v.

PARAMETERS :

============

Range  :: A boolean value which determines the voltage range of the  AIU.

If Range=0 then the voltage range is +/-1.25v.  Any other value for Range

will set the range to +/-2.5v

RETURNS :

=========

Nothing.

SetVoltage(Value, Range)

========================

DESCRIPTION :

=============

Sets the DAC on the AIU to a specified voltage.

PARAMETERS :

============

Value  :: A floating point value representing the voltage to be  sent  to

the DAC. This value is automatically clipped to the limits of the desired

voltage range.

Range ::  A boolean value that determines which range the routine uses to

convert the voltage to a byte.   0=+/-1.25v, and <>0=+/-2.5v.  Note, this

range parameter does NOT change the actual range on the AIU.  It has been

assumed that this range has already been set using the SetRange routine.

RETURNS :

=========

Nothing.

DetectAIU()

===========

DESCRIPTION :

=============

This routine searches all parallel port addresses for the AIU, and sets a

global  variable "PortAddr" which contains the base port address  of  the

parallel  port. This base port address is then used by all of  the  other

routines,  thus this routine MUST be  called  before  any  of  the  other

routines will function. If the AIU is not found on any of the ports, then

the "PortAddr" variable is set to zero.

PARAMETERS :

============

None.

RETURNS :

=========

Nothing.

Final Note :

============

Any queries concerning this project should be directed to :

David L. Jones

TRONNORT Technology

12 Copleland Road

Lethbridge Park NSW 2770

Sydney, Australia

Ph  : +61 2 628-1223

FAX : +61 2 628-1223

EMAIL : dljones@ozemail.com.au

WWW : http://www.ozemail.com.au/~dljones