PC-EPROM Burner, PCEB - EA Version

Copyright(c) 1993/95 TRONNORT Technology.

Version 1.15

by David L. Jones

Disclaimer :

The   complete   PC-EPROM   Burner   (PCEB)   package,   including   the

documentation,  executable & source code,  is provided AS-IS without any

warranty expressed or implied.   It is not guaranteed to work with every

possible hardware combination.   The user assumes full risk for the use,

correct or otherwise of this software package.

Copyright :

The  copyright for this package  is  retained  by  David  L.   Jones  of

TRONNORT Technology, and as such no part of this package shall be copied

or  distributed in any way or form.   The original purchaser is  granted

the right to copy this program onto multiple machines for their own use,

provided  that it cannot be used  by  anyone  other  than  the  original

purchaser.

The  source code is also protected under the applicable copyright  laws,

and  may be modified only for the  purchaser's  own  use.   Modified  or

original copies (compiled or source code) are not to be distributed to a

third party by any means or in any form.

PROGRAM OPERATION MANUAL

==========================

Introduction

==============

This version of the PC-EPROM Burner (PCEB)  software is designed for use

with the Eprom Programmer project described in the September and October

1993 editions of Electronics Australia magazine. The design was by Glenn

Pure.

The PCEB software is designed to work with the project hardware  exactly

as described in the article, and therefore no modifications are required

to utilise the software.

This  is not a compiled or modified version  of  the  GW-BASIC  software

described  in the article.   It is a complete Eprom  Programmer  package

written from the ground up by TRONNORT Technology that will allow you to

Edit,  Program, Read,  and Save virtually any size EPROM from a 2716  to

the  maximum allowable by your programmer design.   (The standard design

will  handle up to 27256's).   The device type library included  in  the

program  allows you to make customised settings for an  Eprom  algorithm

that  is not supported by the built in algorithms.   Programming  pulses

down to 100�sec are supported.

The  program requires a 100% IBM  compatible  machine  with  a  standard

centronics  parallel port and 256K of RAM.   A colour monitor  and  hard

disk are recommended but are not necessary.

My aim when writing this software was to provide most of the features of

the commercial Eprom programmer packages.   A lot of these packages also

have  similar text type interfaces and operability,  which  have  become

some  what of a defacto standard.   Hence,  the  PCEB  software  closely

resembles  this interface and operation.   If you have used a commercial

package  before, then the PCEB program should operate in a very  similar

way.   For first time users,  the program if relatively straight forward

and self explanatory.   Most users will not need to read the operational

section further on, however I recommend that it be read at least once!.

The  vast range of device types and automatic programming modes  of  the

commercial  packages have not been implemented into the  PCEB  software.

This is due to the simplicity of the project hardware design and is  not

a software limitation.

This  software version does NOT support EEPROM's  as  described  in  the

article.

The  reason for not including EEprom support in the software  is because

just  about every manufacturer has a  different  programming  algorithm.

These algorithms cannot be covered with a simple 'generic' algorithm  as

Eproms  can.  This makes writing the software to  cover  many  different

brands extremely complex.   There is not much point in catering for just

one brand!.

The Source Code

=================

The  complete source code has been provided on  disk  to  allow  you  to

modify  the program if need be.   However,  the  program  is  relatively

complex  and it is not recommend that you modify  anything  without  the

proper  knowledge.   The  Eprom  timing  algorithms  are  probably  more

critical than you might expect.  So don't think that changing one little

thing  will not do much!.   Even the slightest change to  the  algorithm

requires  you to retest all the  algorithm  options,  and  check  timing

routines  with a CRO.   I say all of this just to warn you, so don't let

it discourage you from experimenting!.

All  of  the  program  code  is  contained  in  a  single  file   called

'EPROMxxx.PAS'.  Where 'xxx' refers to the version number.  Eg, '115' is

version 1.15

The  entire PCEB program  was  written  in  Borland  (Turbo)  Pascal  7.

Earlier  versions of Turbo Pascal and other brands of  Pascal  have  not

been  tried.   It  is  highly  likely  that  they  will  require   minor

modifications  to the source code.

The  code may not be spaced out as much as it could be,  this is because

Pascal  7 uses that new colour syntax highlighting which makes the  code

really  easy to see within Pascal 7's environment.   I highly  recommend

upgrading to either Borland or Turbo Pascal 7 if you don't already  have

it!.   My apologies also if you do not like everything  being  in  UPPER

CASE, I just prefer it that way !.

I would be very interested to see any modifications that people make, so

that  I can possibly add them to  the  full  product  and  let  everyone

benefit.   I would also welcome comments on the code in general,  except

for the UPPER CASE thing !.

Software Performance

======================

Those  who have used a commercial programmer before may notice that  the

PCEB software is considerably slower on the faster algorithms (0.1ms and

1ms)  than  commercial  units.   This  slower  performance   was   added

deliberately  to ensure that the  software/hardware  combination  is  as

reliable  as possible.   Some aspects  of  the  programmer  design  also

dictated  this deliberate action (Eg,  the RC debounce circuit,  and the

use of the parallel port (it's not as standard as you might think !)).

My  reasoning is that it is better to have a slower  and  more  reliable

programmer,  than one that rips  along  and  gives  you  occasional  bit

errors.   A single bit error on an EPROM can mean that you have to erase

it  and try again (which takes at  least  15-30  minutes).   Also,  this

project  is not intended for a production environment,  and so speed  is

not of much consequence.

For  those who like to live life on the edge,  the program's  performance

can  be improved to a  certain  extent  with  a  command  option  to  be

described later.  Likewise, for the completely 'bit-error' paranoid, the

same option can slow the algorithms down.

For  the slower algorithms (>1ms),  there is almost not decernable speed

difference from commercial units.

Discussion on Limitations

===========================

Due  to a compatibility problem between the programmer  hardware  design

and  some parallel ports,  I recommend leaving the  Vcc  switch  on  the

programmer  permanently set to 5v.   Even  for  the  newer  Eproms  that

recommend  Vcc should be taken to 6v while programming.   This is not as

bad as it seems,  as this higher Vcc voltage is not an important part of

the  programming process.   It is used when verifying the byte after  it

has been programmed, as the higher Vcc voltage merely provides a greater

margin for detecting any errors.

If  you really want to do the job properly,  you would have  to  provide

different  types of 74xx157 chip depending on the type of Eprom you  are

programming.   Therefore, for this design,  there is very little benefit

in reading the data back with Vcc set to 6v.  Generally, I've found that

it is not very reliable with this project.

The PCEB software works very well for both old and new Eproms,  and will

program  the majority of current device types available.   You would  be

unlucky  to get an Eprom that  you  cannot  program  using  one  of  the

available  algorithms obtainable within the Type Library.   The Eproms I

have  tried range in age from 1978 to 1992,  and come from half a  dozen

different  manufacturers.  They were both brand new and  used.   All  of

these  Eproms worked with one of the standard  device  types  selectable

within the Type Library.

Many  others have used the software without any problems what  so  ever,

using all different types of NMOS & CMOS Eproms.  So the design is quite

well proven.

Version  1.1 and above supports devices that require a  100uSec  program

pulse.  Devices that require Prime and Final pulses, as used in the FAST

programming  algorithm are also supported.   As far as I can  tell,  the

following algorithms can be accommodated with the PCEB software:

Standard Generic, FAST, Quick Pulse, SNAP!, FLASHRITE, and PRESTO2.

There are no doubt others that can be implemented with a combination  of

parameters  alterable within the Type  Library.   Most  algorithms  from

different manufacturers are just slight variations of these few,  and in

most  cases are completely identical.   Manufacturers just like  to  use

their own fancy names,  and just rearrange the programming flowcharts in

the data book!.

Usually, the 1ms pulse width can still be used with current devices that

require  a 0.1ms or 0.5ms pulse.   This is because of the way  in  which

manufacturers  specify these limits.   For example,  a manufacturer  may

specify  a program pulse width of 0.1ms +/-5 or 10%.   However, this  is

the required pulse width for that particular manufacturers newly dreamed

up  'wizz-bang'  algorithm,  and  is  usually  not  an  absolute  design

specification.   I have yet to find  a  manufacturer  who  specifies  an

absolute limit for the program pulse length.   I have found that most of

these  Eproms  are  backward  compatible  with  the  older   1ms   pulse

algorithms.

I  have been programming Eproms using commercial  programmers  for  many

years,  and  cannot  remember  ever  having  to  select   a   particular

manufacturers  algorithm for the chip I was  programming.   They  always

worked with the software set to the standard 'generic' algorithm.  There

are  even some brand new Eproms that specify a 0.1ms pulse that did  not

work with the correct manufacturers algorithm, but they worked perfectly

with the generic 1ms pulse!.

In no way am I recommending that the best way to program a device is  to

use  anything other than the manufacturers specifications.   But if  the

programmed  device works and continues to work reliably,  then  does  it

really matter which algorithm you use to program it?. Anyway, if you are

serious  about programming Eproms,  you should  be  using  a  commercial

programmer  specifically designed for the device  you  are  programming.

Also,  this programmer design is very simple and does not exactly  match

even one manufacturers specification.

However,  the PCEB software and EA hardware  are  quite  acceptable  for

occasional programming, and will handle the majority of types available.

Program  Operation

====================

IMPORTANT :

===========

The  PCEB program CANNOT be run from a write protected floppy disk.   Do

NOT turn off write protection on the original PCEB disk.  Either install

it  on a hard drive,  or onto a second working diskette.   Always make a

backup copy of any original disks, and mark them as such.

WARNING:

========

The  length of the program pulse width can change if the  CPU  speed  is

altered  after the program has been loaded.   Generally, this is  not  a

problem  for desktop machines,  just don't touch the TURBO button  after

the program is loaded.  However, some laptop machines dynamically change

CPU speed to save battery power.  For example, the IBM L40sx laptop does

this.   If the software is to be used on one of these laptops, make sure

this  power management feature is turned off.   I do not recommend using

the software under Windows, however I have tried it in a DOS shell under

windows and it did seem to work fine.  But the Windows configuration can

vary  widely between machines.   Try it at you own risk!.  I also do not

recommend  using the DOS 6 POWER program with  this  software,  as  that

program  works by 'stealing' CPU cycles,  which could affect  the  pulse

timing.

Ensure  that the Vpp switch on the programmer is always set to OFF,  and

only  switched ON just before programming is to commence.   This  is  to

ensure that the Eprom is not corrupted in any way.

If  the program is running at  the  main  menu  and  the  programmer  is

connected  to the selected parallel port,  the D0 LED on the  programmer

will  be lit.   All other LED's should be off.   This indicates that the

programmer  is connected and is talking to the software,  although it is

not a guarantee that there are no other problems.

Hardware Requirements :

=======================

Hardware  requirements for the PCEB are very simple.   It will  work  on

virtually any IBM compatible machine,  from a simple 4.77MHz XT right up

to 386/486 machines.   The program operates entirely in text mode, so it

will  work on any monitor from MDA to  VGA.   Monochrome  monitor  users

should  start the program with the 'M' option.   Operation from a floppy

drive  is possible,  however a hard disk will provide  very  significant

performance gains.

The  program has support for all three parallel ports  on  the  PC,  the

address of which is selectable from the main menu.

Again, ensure that the CPU clock speed does not change while the program

is operating.

Installation and Startup :

==========================

An installation batch file called "INSTALL.BAT" will install the program

files  on the specified drive.   Alternatively, all of the files can  be

copied onto any drive or subdirectory.

All  that is required to run the program is  the  main  executable  file

'EPROM115.EXE'.   It also requires enough  disk  space  in  the  current

directory  to store a temp file which will be the same size in bytes  as

the selected Eprom, or a minimum of 32Kb.

The  'EPROM.BAT' batch file starts the program in colour  mode,  or  for

monochrome  monitors the batch file 'EPROMM.BAT' should be  used.   This

just  adds an "M" after the program name,  which can be done manually as

"EPROM M".   This is the first parameter and must be added if the second

parameter  is used.   For colour  operation,  just  make  it  any  other

character than "M".

The second parameter is a number in the range 1 to 65535 that  specifies

a  delay that is  used  in  the  program.   This  delay  is  mainly  for

overcoming  the R/C damping circuit used on the address clock  increment

line  on the programmer.   The default is 50, which should work for most

machines.  As a guide, a 20MHz 386sx machine should work with a value of

ten.   The default value of 50 is used to  allow  for  really  fast  486

machines.   You can decrease the value as long as  the  address  counter

increments correctly, and you should notice a significant improvement in

device  access times.    Eg "EPROM A 10",   selects colour display   and

delay=10.

After startup,  the program loads the configuration file "CONFIG.EPB" if

it exists.  This file contains the last port address, filename and Eprom

type that was selected in a previous session.  If there are any problems

on startup then you can delete this config file and the program will use

default settings.

If  a filename was specified in a previous session,  then that file will

be automatically loaded if it still exists.

After  the config file is loaded,  the program 'Calibrates' the internal

timer.   This  takes  3  seconds,  and  is  machine  independent.    The

calibration value is only used for the 100usec programming pulse.

Buffer File Operation :

=======================

Due  to the programs capability to support device  sizes  up  to  16Mbit

(2Mbyte)  in size,  the program uses a "buffer file" on disk instead  of

using  RAM. This buffer file has a minimum size  of  32Kbytes  which  is

sufficient  to handle up to 256Kbit devices.   If  a  larger  device  is

selected then the buffer will enlarge in 32Kbyte increments.  The buffer

file  is always used although a small device  size  is  selected,  which

could easily fit into memory.

The buffer file 'TEMPFILE.EPB' is not erased when the program is exited,

but it IS erased when the program is started.   So if you forgot to save

the  data before exiting,  you can just rename the  buffer  file  before

starting the program again.

The  only disadvantage to the use of a buffer file is  that  it  is  not

quite  as fast as memory.   However, if the program is operated  from  a

hard disk you can hardly notice the speed degradation.

For  relatively slow hard disks,  it is suggested that a disk cache such

as SMARTDRV.EXE be used. It should be larger than the size of the buffer

file.

The  Main  Screen

===================

All  program operations are selected from the main menu options  located

down the left side of the main screen.   The highlighted letters are the

selection keys for each particular function.

The  right side of the screen contains the parameters of  the  currently

selected  EPROM, as well as the current file name,  directory and amount

of free disk space.

The bottom right displays all of the device address information.   These

are  continually updated while a device is being accessed.   The Current

Address  value is updated in increments of 10,  so that updating it does

not slow down the algorithm.

'Last  Address' contains the  last  address  of  where  a  function  was

stopped.   For example,  if the device failed any  function  then  'Last

Address' would contain the address at which the error occurred.

Note, all the address displays are in Hexadecimal.

Checksum Routine

==================

A checksum of the buffer file for the current address range specified by

the Start/Stop addresses is displayed in the bottom right corner.   This

checksum value is updated after every operation that modifies the data.

For slow machines using large address ranges,  this procedure may take a

few seconds.

The checksum is calculated using the CCITT CRC-16 polynomial.   This may

or  may not be compatible with other commercial programmer software,  as

different formats seem to be used.

The  separate  CHECKSUM.EXE  program  supplied  with  the  package  will

calculate CCITT CRC-16, XMODEM CRC16, and CRC-32 values of the specified

file.   Note that this program only calculates the  CRC  value  for  the

entire file.

Main Menu Functions

=====================

The following options are available for selection from the main menu  by

pressing the corresponding highlighted key :

Auto Program : (A)

==============

Executes the following commands in order without operator intervention:

- Blank Check

- Program

- Compare

If  any of the tests fail then the process  is  stopped  with  an  error

message.   This batch command is generally  used  to  program  an  Eprom

without any fuss.

Program : (P)

=========

You  will first be prompted to start the programming process,  at  which

point the settings (Vpp, Vcc, Device) on the programmer should be set to

the required positions, which are displayed in the Options menu.

The  switch settings displayed on screen are  for  information  purposes

only,  as these are all manual controls on the  Programmer.   Therefore,

the controls can be set to any position without changing the device type

in the software.   The software cannot detect if the switches are set to

the  correct positions before programming begins,  so check the settings

before  proceeding to avoid  incorrectly  programming  or  damaging  the

device.

Once  the settings are correct,  press 'Y' to  start  programming.   The

current address is shown on screen,  and will increment only if the last

byte  verified correctly.   The Eprom is programmed  using  the  current

parameters displayed in the Settings menu.

If  a byte cannot be programmed at the current address after the  number

of  tries specified by the Retry  setting,  an  error  message  will  be

displayed and you will have the option to continue or quit  programming.

Only  continue programming if you are sure that the current byte is  not

important, and you know exactly what you are doing.

If you have specified a start address other than zero, then the software

will   first   skip   to   that   address,   and   will   then     start

programming. Programming will stop at the specified End address.

Blank Check : (B)

=============

Checks if the device is blank,  IE. if every address is set to FF (every

bit  is high).   The program will keep checking until the device Size is

reached  or the ESC key is pressed.     If any byte  is  found   to   be

anything other than FF,  the routine stops and displays the last address

were the error occurred.   Ensure that the Eprom size is the same as the

device  to be read.   The device will only be blank checked between  the

address ranges specified.

Read : (R)

======

You  will first be prompted to commence the Read  operation.   Selecting

'Y' will overwrite the existing buffer file, so make sure you have saved

that  data if need be.   The Eprom contents will then be read  into  the

buffer file.   The data will be read from the specified start address to

the end address, or until the ESC key is pressed.

Verify : (V)

========

This  operation will compare the data contained in  the  buffer  to  the

corresponding  data in the Eprom.   The reason it is called 'Verify'  is

because  the data is read with Vcc set to 6v.   This command will rarely

need  to be used,  but it is provided just in case.  Remember to set the

Vcc  switch to the 6v position,  and back to 5v after completion. Ensure

that  the Vpp switch is set to OFF  before  commencing  this  operation.

This is because the Vpp and Vcc lines are tried together in the  project

hardware,  and this may lead to  the  information  in  the  Eprom  being

corrupted if Vpp is left on.

Again, the start and end address ranges specify the area of the Eprom to

be verified.

Compare : (C)

=========

This command is identical to Verify, except that Vcc is set to 5v.

The contents of the buffer file are compared with the corresponding data

in the Eprom.   If the two bytes at the current address are not the same

then  the operation stops,   and the last   address   were   the   error

occurred is displayed.

Load : (L)

======

The  Load operation will read the contents of the file specified by  the

Filename setting into the buffer file.   If Filename is smaller than the

current Eprom size then the rest of the buffer file is filled with FF's.

If  Filename is larger than the current Eprom size,  data is read  until

the buffer file is full.

The  PCEB software reads and writes files  in  standard  binary  format.

This format is compatible with most compiler software,  so compatibility

should not be a problem.

The  Load operation is not affected by  the  Start/End  address  ranges.

Reading will always commence at the first byte in the file.

Save : (S)

======

The Save command will save the contents of the buffer file into the file

specified  by the Filename setting.   If the file exists  then  you  are

prompted to overwrite it.  If the current Eprom size is smaller than the

buffer  file size,  then the file saved will only be as big as the Eprom

size.   The Save operation is not  affected  by  the  Start/End  address

ranges.

Edit : (E)

======

This is a complete hex editor that allows you to modify any byte in  the

file buffer.   Usually, this editor is only used for quick modifications

to  a file,   and you really must know what you are doing if you  modify

the data in this way.

The  data is automatically saved back to the file buffer when  you  exit

the  editor.  You should always have the data saved in a  file  on  disk

before  you attempt to enter the Editor.   This is  to  ensure  you  can

reload the original data if need be.

The  editor should not be used  for  just  viewing  data.   The  Display

command is provided for doing this safely.

Edit Screen command :

=====================

The  edit screen is divided into 3 columns.   The left most column shows

the  address of the first  byte  on  that  line.   This  address  is  in

hexadecimal.

The  middle column contains rows of eight data bytes  displayed  in  hex

format.   The address of each byte is the address display in  the  first

column plus the offset display at the top of the screen.

The rightmost column contains the equivalent ASCII character for each of

the  hexadecimal data bytes in the second column.    This can be  useful

for seeing text messages embedded inside the file.

The  F7/F8 function keys select different 'blocks' of data.   Each block

is  32Kbyte in size,  therefore this command will only works for  device

sizes greater than 32Kbyte.

The  PgUp/PgDn keys scroll the screen in 256  byte  'pages'.   Only  one

'page' of data is displayed on screen at any one time.

The Home and End keys take you to the first and last page of data in the

current block.

A  cursor is initially displayed over the first data  byte  in  the  HEX

window.  This cursor can be moved with the cursor keys over the hex data

displayed on the current page.

The TAB key swaps the cursor between the HEX and ASCII windows.

When  the cursor is in the HEX window,  you can edit the data byte under

the cursor by entering two hex digits.   Eg, to change the byte to '0F',

press '0' then 'F'. You do NOT have to hit Enter.

When  in the ASCII window,  the byte selected by the cursor position can

be modified by pressing any ASCII key on the keyboard.  This will change

the current byte to that character.

The  F9 and F10 function keys will decrement or  increment  the  current

byte one value at a time, regardless of which window you are in.

Display : (D)

=========

The display feature is identical the Edit command.   The only difference

being  that you are not allowed to edit the data in any way.   Use  this

command  if you only want to view the data,  as there is  no  chance  of

accidentally changing it.

Refer  to the Edit section for information on how  to  use  the  display

features.

Filename : (F)

==========

Allows  you to change the current filename,  which is used for the  Load

and  Save commands.   You can include a drive and path name in  standard

DOS format.

Directory : (\)

===========

Provides a directory of the current path.  The third column shows if the

file  is an exact binary multiple.    Other than that,   it  is   almost

identical to a standard DOS Directory.

Type Library : (T)

==============

The  type library provides a means to store  parameter  information  for

different  types of devices.   It is also the only  way  to  modify  the

current Eprom settings.

The  program comes with about  15  different  types  of  Eproms  already

programmed.   You can overwrite these if you wish, but it is best to Add

a  new device and experiment with that.   The default  settings  covered

every  Eprom that I could find,  you would be hard pressed  to  find  an

Eprom that won't work with one the these settings.

The screen is divided into 9 columns.  The left most column displays the

current  device number.    There can be a maximum of 500 devices. Any of

the six right most numerical columns can be changed by  highlighting the

required  one and using the +/- keys to change  the  value.    The  next

column contains the device description.   It usually contains the device

type number and any associated comments.  This description can be edited

by highlighting the description and pressing 'E'.

The 'Bytes' column displays the device size in number of bits.  This can

be  changed from 16Kbit to 16Mbit.    A  2716   Eprom   corresponds   to

16Kbits and so forth.

The 'Pulse' column specifies the duration of the program pulse in  milli

seconds.   This is adjustable from 1ms to 100ms. If it is set to 0, then

the program pulse width will be 100 micro seconds.

'Vcc'  is one of the most important settings,  and must be set correctly

depending on the device being programmed.  'Vcc' can be set to either 5v

or 6v, here is an explanation of each :

With  'Vcc' set to 5v,  the data is verified during programming at Vcc=5

volts.   The data is still programmed with Vcc=6v, but as I explained in

the discussion,  this is not desirable.  Therefore the Vcc switch on the

unit  should be set to 5v when this setting  is  used.     The   program

pulse will also be turned off after each byte is programmed, and this is

the  only way to program older 2716 and 2732 Eproms.   Some  2764's  and

higher  also require this.   The only disadvantage is that  the  minimum

pulse  width must be at least 1ms.   This is due to  problems  with  the

design, as the Vpp line takes something like 300us to turn off.

With  'Vcc' set to 6v,  the Vpp and Vcc lines are  kept  permanently  on

during programming.  This allows a faster programming pulse (<1ms) to be

used.   This setting will only work for the newer 2764 and above Eproms.

The Vcc switch on the unit should be kept at 5v when using this setting,

otherwise the data tends to be verified incorrectly.

The  Vpp setting does not  have  any  affect  in  this  version  of  the

software,  as this is a manual control on the unit.   It is  good  as  a

reminder of the voltage that the current device requires.

The  'Try' setting specifies the number of times the program should  try

to  program the byte into the current Eprom address  if  there  are  any

errors.   Some Eproms do not program properly on the  first  pulse,  but

they  may still work perfectly after a few more attempts.   The  default

setting is 10.   If the byte does not program after the number of retrys

specified,  the program process is  stopped  and  an  error  message  is

displayed.

If 'Final' is set to Zero,  then no Final program pulse is added after a

byte  has verified.   If 'Final' is greater than zero,  then  the  Final

program  pulse  will  last  for  the  number  of  Retrys  needed  during

programming multiplied by the 'Final' setting.  This setting is normally

only used with the old style FAST algorithm, and is not usually needed.

The  'Alg' setting determines what 'Algorithm' is used  to  program  the

Eprom/EEprom. The different options are as follows :

Algorithm  0 is used for all Eproms,  and is a universal algorithm  that

can be customised by the other settings.

Algorithm  1 is used for EEproms.   In this  version  of  the  software,

EEproms are NOT supported.  At the moment, the only difference with this

setting is that the Vpp line is turned off during programming.  It might

be good for something!.

To  add a new device to the type library,  press 'A' and a new device is

added using the default values. You can then edit the device.

The type library is only saved to disk if you press 'S'.  This means you

can make temporary changes while the program is running,  and as long as

you  don't save the changes the Library will restore itself  next   time

the program is loaded.

To  select a device,  highlight any column of the  required  device  and

press 'ENTER'.   This will create a new buffer file, and therefore erase

the  old one.   Any information in the buffer file that was  not  saved,

will  be lost.   This is also a way to clear the buffer contents back to

FF.

Use the PgUp/PgDn keys to scroll through the list of devices.

ESC will exit the type library without selecting a device.

Start/End Address : ([])

===================

The  Start and End addresses allows you to specify a range in  which  to

access  a certain part of the Eprom.   These  settings  affect  Program,

Read,  Compare/Verify, and blank check operations. If you have the start

address set higher than zero,  then the first address in the buffer file

will  correspond to that address.   In other words,  if you read in  the

device  contents, the data will be written into the buffer  starting  at

the first address.

These parameters provide powerful flexibility in manually editing  parts

of  an Eprom.   The settings should not normally be altered,  unless you

know  exactly what you are doing.   Any professional programmer software

provides  this function,  and this PCEB software is no  exception.   For

example,   you can Load a small file into the buffer and then program it

into any Start address in the Eprom.

The  End address  cannot  be  larger  than  the  device  size,  and  the

operations  described above will stop when the End address  is  reached.

All  of these operations will first skip to the  Start  address  without

carrying out the corresponding operation.

The  start  and  stop  addresses  do  not  affect  file  Load  and  Save

operations, as this is the preferred method of most commercial packages.

Hex Address : (H)

=============

Allows  you to specify the Parallel port address that the programmer  is

connected too.

378 is the address of LPT1 if you don't have a monochrome/printer  card.

It is the port that most people will need to use.

278 is the address of LPT2 if no monochrome card is used.

3BC  will always be the address of the parallel port on one of  the  old

MDA/Hercules  video cards.   This port,  if available is  always  called

LPT1, and is used on many laptop and older XT machines.

If the unit does not seem to work, then try all 3 addresses.

Quit  : (Q)

======

Quit  the program and returns to DOS.   It does not prompt  you  if  the

current buffer file has not been saved.  See the Buffer File section for

more information.

Trouble Shooting

==================

If  you cannot get the programmer or software to work then  these  notes

may help:

1) Seems to work, but gives occasional errors:

This  problem may be due to a long  parallel  cable  connecting  to  the

programmer.  I suggest mounting a DB25 male connector on the side of the

programmer,  and using a ribbon cable with IDC connectors at both  ends.

You can even use the same cable as my DSOA project (EA,  Feb.93). Try to

keep the cable as short as possible,  preferably under 2 metres. Even if

you  aren't having any problems,  and IDC cable is much better than  the

permanent cable described in the article.  This cable also provides some

ground shielding between conductors.

You may have also selected the incorrect device Type.   Try changing the

Vcc setting in the type library if you are having problems  programming.

The  Vcc setting in the software is very important,  as it determines if

Vpp is turned off after each pulse.

If  you have a funny Eprom,  then you may need to  experiment  with  the

programming settings in the Type Library.   Don't overwrite the existing

default settings, add a new device and experiment with that.

Some  Eproms can draw considerable current when programming,  hence  you

should be using a suitably rated plug pack transformer.   A 12v AC 300ma

5W plugpack is adequate for my unit.

2) Occasional verify errors during programming:

Set  the Vcc switch on the unit to 5v,  as I've found the programmer has

problems reading back at 6v.

3) Doesn't seem to work at all:

You may have the wrong port address selected.  Try the other settings by

pressing 'H'.

Version History

=================

Version 1.0b - First beta version. Monochrome interface.

Version 1.0 - First full version.   Colour interface.  Start/End address

support.

Version 1.1 - Added 0.1ms and final pulse support.   Changed programming

algorithm to be more universal.  Doubled the speed of most

Eprom  access operations  without  affecting  reliability,

(just faster screen updating).

Version  1.11 - TAB function in hex editor added.   Automatic loading of

last  selected file on program  start.   Changed  a  few

messages.

Version  1.12 - Properly restores cursor after exit.   A few other minor

things fixed.

Version 1.13 - Added CRC-16 checksum routine.   The program will now not

load  if write protect is on,  which caused wierd effects

(hanging!)  on previous versions.

Message routine clears previous message first.

Version 1.14 - Program will now properly initialise text mode on program

start.   On previous versions,  the programs  text  color

attributes  would not initialise properly if the  program

was  started  from  within  some  shell   programs   (Eg,

XtreeGold in 'X' mode (not 'Alt-X')).

Version 1.15 - Added new algorithm (#2 in type library), to cater for

different brands on 2716 Eproms. Also added a new 2716

device to the type library to use this algorithm.

Current Address now displays correctly.

Option for "Nonstop" programming is now given when a

programming error occurs, which tells the software to

ignore all future errors and continue programming.

Final Word

============

I welcome any comments on the program and/or source code,  and any ideas

for  improvements.  If you have any problems with the software  and  get

completely  stuck then do not hesitate to ask for advice.   Be  sure  to

return  the product registration form included with the package.   There

is  also  an  on-disk  registration  form  if  one  was  not   included.

Registration  will ensure that  you  receive  all  the  latest  info  on

upgrades and new designs.  Send all correspondence to :

David L. Jones

TRONNORT Technology (Now "Advanced R&D Solutions")

12 Copeland Road

Lethbridge Park NSW 2770

Ph/FAX (02) 628-1223 (Poll FAX for latest info/pricing)

September 19th, 1995.