PC-Logic Analyser (PCLA)

Revision 1.0

Document last updated : 27/8/96

(c)1996 David L. Jones - TRONNORT Technology

DISCLAIMER :

The  PCLA program and all associated files are  provided  AS-IS  without  any

warranty expressed or implied.   Whilst every effort has been made to  ensure

the  program works as described,   it has only been tested with  the author's

prototype PCLA units and associated computer systems.   Due  to  the enormous

range  of PC configurations and potential variations with other  PCLA  units,

the  program cannot be   guaranteed   to   work   as   described   with   all

possible software and hardware variations.

The  PCLA program and all associated files and  documentation  are  Copyright

(c)1996  TRONNORT Technology.  No part is to be copied or distributed in  any

form or by any means.

INTRODUCTION :

==============

The PC-Logic Analyser (or PCLA) program is designed to work with the PC-Based

logic analyser described in Electronics Australia magazine.  The program will

work  with the  hardware  exactly  as  described  in  EA,   unless  described

otherwise later in this document.

SYSTEM REQUIREMENTS :

=====================

The program requires a 100% DOS compatible 386 PC with 640KB of base  memory,

a  VGA compatible  colour  monitor,  and  a  standard  centronics  compatible

parallel port.  A fast 486 with a hard disk is recommended.  The program will

also support any DOS compatible text based printer.

The  program  has  been  designed  for  operation  under  a   Real-Mode   DOS

environment.  It has not been tested under Windows,  Windows 95,  Windows NT,

OS/2  etc... It is recommended that the program  be  run  only  under  a  DOS

compatible  environment, as other resident operating systems,  GUI's etc, may

conflict with the low level calls used by the PCLA program.

If  operation is necessary under a  Windows  type  environment,  then  it  is

recommended  to run the  program  in  Full-Screen  mode,  and  not  within  a

graphical window.

FEATURES IN VERSION 1.0 :

=========================

Version 1.0 is the first release of the PCLA program. Some key features are :

- Full support for all PCLA hardware features

- 32 channel waveform display with zoom/expand capability

- Disassembly display which displays Address, HEX, Decimal, and ASCII data.

Useful for decoding microprocessor buses.

- Ability to save waveform and disassembly displays to a text file, PCX

format graphics file, or print to a text based printer.

- Support for simultaneous operation of a printer on a second parallel port

PROGRAM INSTALLATION :

======================

The program can be installed by either running the INSTALL.BAT batch file  on

the  distribution disk,  or by copying all of the files from the disk into  a

directory on your hard drive.

The  program only requires the main PCLA10.EXE file for  complete  operation,

and  can operate from any directory.   Although the current directory will be

used as the default location for saving and loading of files.

Program startup parameters are stored in the PCLA.CFG file,  which is created

automatically  when the program terminates.  The .CFG file is not present  at

program start, then the program will use default values.

PROGRAM OPERATION :

===================

To  run the program,  simply execute the PCLA.BAT file which loads  the  main

executable file.  If a VGA compatible screen is not detected then the program

will abort with an error message.

If  everything loads fine,  then you will be presented with the main  screen,

from  which you can control all main aspects of the program.  All of the PCLA

hardware  control options such as clock and triggering can  only  be  changed

from the main menu.

The  channel control box is displayed on the left side of the  screen.   This

shows  the current settings for all 32 channels,  including, trigger,  label,

and data type indicators.

The main screen contains the following options :

- F1 Waveform Display : Invokes the waveform display (see later).

- F2 Disassembly Display : Invokes the waveform display (see later).

- F3 Start Aquisition :

Initialises  the PCLA hardware and starts data aquisition using  the  current

settings  displayed at the main menu.  The program will first download all of

the current trigger and clock settings to the PCLA which may take a second or

two.

The program will then initiate pre-trigger sampling on the PCLA, and then sit

and  wait for an end of buffer signal from the  PCLA.  Pressing ESC  at  this

point will return you to the main menu without any data being retrieved  from

the PCLA.

If  an end of buffer signal is received,  the program will start to  retrieve

the sampled data from the PCLA.   This may take anywhere from 5 seconds for a

fast  486 to over a minute for a slower 386sx.   You  can  press  ESC  during

retrieval to return to the main menu,  but this will corrupt the current data

in memory.

- F4 Edit Channel Data :

Allows editing of the channel control data in the box on the left side of the

screen.   A red cursor will appear over the top left box which can  be  moved

with the arrow keys.  Once the cursor is over the box that you wish to change

use the indicated key to change the contents of that box.

The  USED setting can be toggled on and off,  which allows each channel to be

turned  on and off.   Turning a  channel  off  only  means  that  it  is  not

displayed,  the channel is still sampled and retrieved.  A cross in  the  box

indicates ON.

It is recommended to turn all unused channels to OFF.  This will speed up the

waveform  display, and will also prevent these  unused  channels  from  being

printed  or saved to the text file.  Press  the  SPACE  BAR  to  toggle  this

setting.

The  TRIG setting can also be toggled ON and OFF.   Setting a channel  to  ON

means  that this channel will be used  as  part  of  the  trigger  word  when

internal triggering is selected. Setting the channel to OFF means the channel

will  be ignored by the trigger circuit.   Press the SPACE BAR to toggle this

setting.

The  TYPE setting is used for the disassembly display,  and indicates  wether

the channel is a DATA or ADDRESS line. Each channel can be set to a DATA line

from  D0 to D31,  or an ADDRESS line from A0-A31.  Use the plus/minus keys to

change  the value.  A blank setting means the channel is not used as part  of

the disassembly calculation.

There  is nothing to stop you from having multiple channels set to  the  same

DATA or ADDRESS number.  However, there is no need for this, and it will only

upset  the disassembly calculation.  In fact,  if multiple channels have  the

same  DATA  or ADDRESS number,   then the routine will interpret  this  as  a

logical  "OR"  function and calculate the number   accordingly.   So   double

check the settings before exiting.

Not all of the DATA and ADDRESS lines have to be specified.  For example, you

can  only specify D1,  D5, and D17, and the routine will automatically assume

the rest of the data lines are zero. The same goes for the ADDRESS lines.

The  last  field  is  for  the  channel  LABEL.   Each  channel  can  have  a

corresponding user defined 8 character label which can be used to allow  easy

identification of each channel.  For example, if you have channel 1 connected

to the R/W line of a chip,  then you might label channel 1 "R/W" or something

similar.   It pays to take the time to label each channel before use, as this

will  prevent hair loss when you forget which channel you connected to  which

line.  And believe me,  you WILL forget  !.  (Murphy's  first  law  of  Logic

Analysis).

- F5 Trigger Source :  Allows selection between Internal and External trigger

on  the PCLA.   The current setting is displayed next  to  the  menu  option.

Selecting Internal trigger will automatically change the Trigger Polarity  to

Positive (a requirement of the PCLA hardware).

-  F6 Trigger Polarity :  Allows  selection  between  Positive  and  Negative

trigger  polarity on the PCLA when  External  Triggering  is  selected.  This

remains Positive when Internal triggering is selected.

-  F7 Trigger Delay :  Allows selection of 0,2,4,  or 8  cycles  for  trigger

delay.  Trigger delay means that the internal or external trigger signal must

be present for at least 0,2,4, or 8 clock cycles before the trigger signal is

recognised by the PCLA hardware.

Trigger delay can be used to prevent spurious trigger signals from triggering

the  PCLA. But this may be exactly what you want to try and detect,  so check

for the correct setting before use.

-  F8 Clock Select :  Selects the sample clock rate used  by  the  PCLA.  The

options  are 40MHz,  20MHz, 10MHz,  5MHz, 1MHz,  100KHz, 10KHz,  and EXTERNAL

CLOCK.  If set to External Clock,  the PCLA is said to be operating in  STATE

ANALYSIS (or ASYNCHRONOUS)  mode. Any of the other settings will put the PCLA

into TIMING (or SYNCHRONOUS) mode.

When external clock is selected,  the PCLA requires an external clock via the

input connector to latch all of the data. The clock can be of almost any duty

cycle  and frequency up to 40MHz.   It does not even need to  be  continuous.

However,  the PCLA requires at least 32768 cycles to fill it's buffer.  16384

of  these cycles must occur before the 3 seconds allowed for by the  software

before triggering occurs.  This is to ensure that at least half of the RAM is

filled with pre-trigger data before triggering occurs.

-  F9 Clock Polarity :  Sets the PCLA sample  clock  to  either  Positive  or

Negative triggering.   This is very important in STATE (external clock) mode,

as  the correct data has to be present when the sampling occurs.   If you are

having problems with data corruption, then try changing this setting.

- (L)  Load Data : Loads a previously saved ".LA" data file. You are asked to

enter  a filename and optional path,  and if the file is found,  it  will  be

loaded into memory overwriting the current data and settings.

-  (S)  Save  Data  :  Saves  all  current  data,  channel  information,  and

clock/trigger settings into a ".LA" data file.  Each ".LA" data file is 128KB

in  size. You will be prompted for a filename and optional path,  but do  not

include the ".LA" extension.

-  (A) Port Address :  Selects which port the PCLA is attached to.  The three

standard  parallel port addresses are 378h,  278h, and 3BCh.   If you have  a

desktop PC then the first parallel port is generally at address 378h, and the

second  at address 278h.  3BCh is used on many laptops and older style  PC's.

This  setting is saved in the config file for the next time  the  program  is

loaded.

- (P)  Printer Port : Sets the port that the print routine will write to. The

options  are LPT1,  LPT2, or LPT3. If you only have one parallel port then it

will always be LPT1.   If you want to run the printer at the same time as the

PCLA, it is recommended that you have a second parallel port. A cheap $10 I/O

card  can be set to give a second parallel port.   However, you can swap  the

PCLA  and printer on the same port.  This setting is saved in the config file

for the next time the program is loaded.

- (Q) Quit : Quits the program and saves the current settings in the PCLA.CFG

configuration file.

WAVEFORM DISPLAY :

==================

Selecting  Waveform Display from the main menu will  bring  up  the  Waveform

Display.   This shows the waveforms for all 32 channels  simultaneously.   By

default,  50 samples are displayed on  the  screen  with  the  trigger  point

(offset 0) in the middle of the screen.

The first sample displayed will be the same as the last position selected  in

the waveform or disassembly views.

Each  channel will have a waveform corresponding the current data  in  memory

for  that channel.   The program will initially display  random  data.   Each

waveform  is  shown  next  to  the  corresponding  channel  in  the   channel

information box. A LOW is shown in gray, and a HIGH is shown above it in RED.

Each  sample point is shown by an inverse dot on the waveform.   By  default,

all waveforms are shown.

Below  the waveform area is the offset address for each sample.   The trigger

point  is given the reference number zero,  and any sample after the  trigger

point is given a positive number, and each sample before the trigger point is

given a negative number. A yellow trigger line will be shown in the middle of

the sample at offset address zero.

The bottom of the display shows the current cursor locations, time per sample

(from the clock rate), and the time difference between the cursors (if any).

The  waveform window can be scrolled one sample at a time using the left  and

right  cursor keys,  and also panned one screen full at a time using the  "["

and  "]" keys.  You can instantly move to the beginning or end  of  the  data

buffer using the HOME and END keys.

Two cursors are available for measuring time differences between two  points.

Pressing  "C" will  show  and  hide  the  cursors.  Hiding  the  cursor  will

automatically reset their locations. The cursors can be moved using the "<>"

and "()" keys.

Pressing  "T" will return you to the trigger point from  anywhere  within  the

buffer.

The number of samples displayed on the screen can be changed from the default

50  to either 10,25,  or 100 using the "Z" and "X" keys.  50 samples provides

the  best compromise between clarity and amount  of  data  shown.   When  100

samples  are displayed,  there is not  enough  room  to  display  the  offset

numbers.

The waveform screen contents can be saved in a number of different ways :

Pressing "S" will save the waveforms and channel information box to an  ASCII

text  file with the extension  ".CAP".   Header  information  containing  the

date/time,  and sampling information are also added.   The cursors and offset

numbers  are  NOT saved using this option.     Only waveforms that  are  USED

will be saved.   You will  be prompted for  a  filename  and   optional path,

but  don't  enter  the extension.

Pressing "P" will generate the same information as above, but instead send it

to the printer port specified in the main menu.   The routine will print to a

standard  DOS LPT device,  so this should be able to be redirected with other

programs if need be.

Pressing "G" will grab the entire screen contents (including the cursors) and

save  the image to a PCX format picture  file  named  "PCLA_WAV.PCX"  in  the

current directory. There is no option to save to a different filename.

Press ESC to return to the main menu at any time.

DISASSEMBLY DISPLAY :

=====================

The  disassembly display is used for decoding data and address buses used  in

microprocessor based designs. The idea is to capture data on the lines of the

data and/or address bus,  and decode these to provide a hexadecimal, decimal,

or ASCII display of what is happening at each sample. This allows you to more

readily  identify problems,  than would be the case if you  had  to  manually

translate the waveform display into meaningful data.

In  order for the disassembly  display  to  work,  you  must  define  certain

channels  to be either data or address lines by  setting  the  required  type

within the "TYPE" field in the channel information box.

The first sample displayed will be the same as the last position selected  in

the waveform or disassembly views.

When you enter the disassembly display,  you will see 5 columns of data which

can be scrolled through the entire contents of the data buffer.

The  left most column is the offset location,  which corresponds to the  same

offset number in the waveform display. Once again, the trigger point is given

the  reference number zero.  The screen initially appears  with  the  trigger

point in the middle of the screen.

The  other columns contain the decoded Address,  Hex Data,  Decimal Data  and

ASCII data displays respectively.   The Hex, decimal, and ASCII displays show

identical  information decoded from the Data  type  information.   The  ASCII

display only works on the first 8 data bits,  and so will only show one ASCII

character.

You  can scroll the display using the up and down arrow keys,  and  move  one

page at a time using the PgUp and PgDn keys.  The Home and End keys will take

you  to the beginning and end of the data buffer.  Like the waveform display,

pressing "T" will get you back to the trigger point.

The  screen information can be saved in exactly  the  same  way  as  for  the

waveform display, by using the "P", "S", and "G" keys.

Pressing "S" will save the current screen to an ASCII text file with a header

containing  the date/time and clock/trigger data.   Unlike the waveform view,

the  channel information is not  saved  in  disassembly  mode.  You  will  be

prompted for the filename and path.

Pressing  "P" will generate the same information as above,  but will  instead

send the text data to a printer at the port defined at the main menu.

Pressing  "G" will generate and save a  PCX  format  graphics  image  of  the

current screen in the file "PCLA_ASS.PCX" in the current directory.  There is

no option to change the filename or path.

PRINTING TIPS :

===============

Both  the waveform and disassembly views can be printed to any  printer  that

can  accept 8 bit ASCII data.   This will include virtually  all  dot  matrix

printers,  be they EPSON or IBM compatible.   Although, some printers may not

have support for 8 bit ASCII by default. In this case, you may have to change

the  printer configuration,  or alternatively save the screen to a text  file

and print it out with a separate program.

The  same goes for many laser printers which  don't  select  8-bit  ASCII  by

default,  in which case you will have to  change  the  printer  configuration

using DIP switches or a configuration program.

If you are printing all 32 channels in waveform mode, then the image will run

onto  two pages,  as the program needs two lines to print each waveform.  You

can avoid this by turning off channels that are not needed,  or alternatively

setting your printer up for compressed printing before you print.

Many new laser printers are not directly DOS compatible.   This includes most

low cost "GDI" lasers, which use Windows and your PC to do all the processing

for  printing. With this type of printer,  you have no option but to run  the

PCLA program in a DOS shell under Windows.

PROBLEMS :

==========

If  you have any problems with the software and require assistance,  I can be

contacted by MAIL,  FAX or EMAIL.  If sending by mail, please include a reply

paid envelope to speed the reply.

Send all enquiries to :

David L. Jones

TRONNORT Technology

12 Copeland Road

Lethbridge Park NSW 2770

Ph/FAX : (02) 9-628-1223

EMAIL : dljones@ozemail.com.au